# 9 LED Cluster V2b Testing

# General

- Works with LED Sculpture Controller (Teensy) V1

- Vcc ~= 4.0 V

- Vreg (SPI) = 3.3 V

- The far red is very faint, perhaps not useful

- The Oslon red is more of an orange red, which could be fine.

- With both far red lines driven at 65000 (brightness=127), R8 reaches 49°C, U2 reaches 41°C

# Soldering

- The failures on SN #3 were both due to apparent solder bridges which appeared to be caused by too much solder paste.

- It is possible to remove LEDs with the hot air rework tool, but it seems likely to damage the top plastic of the LED. I think bottom side heat might be better, or both. I somewhat doubt that I could solder on a new LED with the hot air gun unless I also used bottom side heat.

# SN #1

- Yellow LED does not work

# SN #2

- Failed to power up on first attempt, then powered up

- Red LED does not work

# SN #3

- Cyan and white do not work - removed