# 9 LED Cluster – Revision Notes

# Revision v1 – 2025-02-01

- First revision

# Revision v2 – 2025-09-22

## General

- No: Add some test points.

- DONE: Remove J1 and J2 pads from the paste mask.

- DONE: Move pad labels to the bottom so that I can make the board narrower. Or remove altogether.

- DONE: Document the mounting hole sizes on the schematic.

- DONE: Remove thermal reliefs for the mounting holes to the ground plane.

- DONE: Renumber designators.

- DONE: Try with 0603 to see if it makes the board larger.

- DONE: Change to 4-layer PCB.

- No: Consider metal core PCB

- Future: Add fiducials

- No: Try single-layer PCB design with zero-ohm jumpers instead of bottom layer tracks. This might allow an inexpensive metal core PCB and better heatsinking.

- DONE: Make room for a fourth corner hole.

- Future: Consider adding some thermistors to measure temperatures. Perhaps with I2C DAQ.

## Power Supply

- DONE: Update Vreg feedback calcs. R5 and R6 are reversed.

- DONE: Change to 4.0 V output.

- Skip for now: Can I find a buck converter with a higher switching frequency and a smaller inductor? – Yes, higher freq (2.0 MHz vs 1.1 MHz), the inductor drops from 15μH to 10μH, but ends up the same size. Consider integrated converter.

- Skip for now: Consider DC-DC module like TI TPSM33615.

- DONE: Replace R7 with a ferrite bead.

- Future: Consider one FB at J1 and one at J2. Or use CM choke at each end.

  - Murata BLM15AG601SH1D 0402, 600Ω @ 100 MHz, DCR=0.6Ω

- DONE: Route the Vin track on the third layer and further from the buck regulator to avoid noise coupling.

- Somewhat done: Surround the buck regulator switching nodes with ground.

- DONE: Consider an isolated ground plane with single connection for the buck regulator.

- Future: Test conducted RF from the regulator onto the Vin and Vout leads.

- DONE: Changed buck output cap from 1206 to 0805.

- DONE: Test at Vcc = 4.0 and 4.25 V to see how it affects the heating. Make sure we won’t overvoltage the SPI inputs (Vcc+0.6 abs max). Use the same Vcc for the level shifter.

  - Vcc = 4.0 V works well

- Future: Consider UVLO on the power supply

- DONE: Protect against reverse polarity connection, if not too difficult.

  - Diotec SKM14 Schottky, 40V, 1A, SOD-123FL, 350mV@1A

## LED Driver

- DONE: Disconnect VREG from VCC so the SPI levels will be 3.3V. Add a 1.0μF bypass cap on VREG.

- Future: Protect SDTI and SCKI from exceeding Vcc+0.6, perhaps with Schottky diodes or SOT-23 input protection diodes to Vcc. Maybe also series current limiting resistors.

- Future: Consider 3-terminal feed through capacitors. Maybe replace five 0.22uF with 1-2 three-terminal caps.

  - https://product.tdk.com/en/techlibrary/solutionguide/yff-series.html

- DONE: Add dropping diodes or resistors for lower Vf diodes, like red and amber. This will reduce the heating in the TLC5971. These could be populated with zero-ohm jumpers when not needed. It looks like 0603 resistors would be best, 16.0 or 16.5 ohm for 60 mA, only for LEDs with ~2.0 Vf.

  - Rohm ESR03EZPF16R5, 16.5Ω, 1%, 0.25W, 100 ppm/°C, thick film, 0603

  - Panasonic ERJ-PA3F16R0V, 16.0Ω, 1%, 0.33W, 100 ppm/°C, thick film, 0603

  - Vishay Dale CRCW06038R20FKEAHP, 8.2Ω, 1%, 0.33W, 100 ppm/°C, thick film, 0603

- Skip: Consider dual drivers to allow 2X the power. Would likely overheat the PCB and LEDs.

## LEDs

- DONE: Center LEDs on the PCB and align the corner holes to be symmetrical.

- DONE: Larger LED cathode mark in copper (it’s not really visible). Or remove and just use silkscreen.

- DONE: For the Luxeon CZ LEDs, make LED pads slightly taller, maybe 0.1 to 0.2 mm on each end (right now, they are fully covered on both ends).

- DONE: Consider LED diffuser mounting holes.

- DONE: Leave a clear margin around the LEDs for light shaping / diffusing

- DONE: Leave ~1mm between LEDs for barrier to block blue/violet from white phosphors. This may not be needed if I use flat top LEDs (no lens).

- DONE: Use a mixture of OSLON Pure 1414, Cree XE-G, and Luxeon CZ LED since we are only driving 60 mA to 120 mA. This will save money and space.

- DONE: Change to a shorter wavelength violet (410 nm instead of 425 nm)

# Revision v2b – 2025-10-08

## General

- Remove corner holes from the solder paste stencil.

- Adjust solder paste expansion.

- Adjust solder paste stencil size and thickness.

# Revision v3 – tbd

## Future

- Add fiducials

## LED List

- Violet 410 (XE-G)

- Blue PC 460 (OSLON)

- Cyan 500 (XE-G)

- Green 525 (OSLON)

- Yellow PC 590 (OSLON)

- Red 620 (OSLON)

- Photo Red 660 (XE-G)

- Far Red 735 (Luxeon)

- White (OSLON)

| Yellow PC | Red | Blue PC |
| --- | --- | --- |
| White | Photo Red | Cyan |
| Green | Far Red | Violet |